# Electrical and Computer Engineering Department

# University of Puerto Rico at Mayagüez

# COMMUNICATIONS THEORY I

MATLAB CLASS ASSIGNMENT 01 (MCA01)

Modeling a Double Sideband Suppressed Carrier Communications System for Underwater Analog and Digital Acoustic Communications

# By

# <SXXXGPYY>

# For

Prof. Domingo Antonio Rodríguez

Electrical and Computer Engineering Department

University of Puerto Rico at Mayagüez

domingo.rodriguez1@upr.edu

<260825>

INTRODUCTION

The main objective of this MCA01 is to model a double sideband suppressed carrier communications system (DSB-SC) for the communication of speech  and DTMF signals in an underwater medium. The maximum frequency content of all these signals is assumed to be FV=4000Hz and the “one-sided” channel bandwidth is said to be e B=14000Hz. A receiver low-pass filter must be designed to recover the signal xr(t)≈s(t) (see Fig. 3).

The channel itself is modeled as a composition of two basic systems, an ideal, linear phase, low-pass filter TC, with cut-off frequency, f=FL,  and a noise signal summing system, TN, where the channel underwater acoustics noise signal, n(t), is added to the output of the linear phase filter TC. This DSB-SC communications channel is given the name TCN=TAM. Thus, it is given by cascading the sub-systems,  TC and TN (see Fig. 2).

The overall sampling frequency of the DSB-SC communications system must be set to a value greater than twice the maximum frequency (Nyquist-Shannon sampling theorem) content of the output of the demodulator. That is, FS>2(2fc+FV), where fc is the carrier frequency of the modulator (demodulator) and must be fc=8640. The input signal xm(t) to the modulator is the sum of a “wanted” speech signal s(t) and an “unwanted” interference signal g(t). After the demodulator, an ideal, linear phase, low-pass filter, TL is used, with cut-off frequency FM=4000Hz, to recover the  speech or voice signal or DTMF signal s(t) (see Fig. 1).

DESIGN PROJECT:

Each group must work together modifying and running the DSB-SC simulation to complete and submit the MCA01.

01.- Access and review all hyperlinks provided.

02.- Run the given program, study all the graphs, and

observe that the ideal, linear phase, band-pass filter does eliminate given interference signal g(t).

gt=cos2π4400t+cos2π4900t+cos2π5400t

03.- Verify that channel filter is an “all pass” : hCt=δt.

04.- Verify that receiver’s band-pass filter is: hRt=δt.

05.- Change the carrier frequency to fc=8640.

06.- Increase value of noise parameter factor nATT by 40 dB, from 1224000 to 122.4  to obtain about -4 dB SNR decrease in the channel output ycot.

07.- A linear phase, low-pass filter, TL, with cut-off frequency FV=4000Hz, for the receiver was designed. A figure of merit for this assignment, is the  time-delay, Td=ML-12Ts, where ML is the order of the filter and Ts=1Fs is the overall sampling time or sampling period of the system. For this work, in order to enhance impulse response function, set the order to ML=Nlow=1323.

08.- Generate: 1) magnitude of the spectrum of the output signal yco(t) of the channel, 2) the magnitude of the spectrum of the output signal yd(t) of the demodulator, 3) the impulse response function hLt of the receiver’s low pass filter, 4) the magnitude of its spectrum, HLf, 5) the receiver’s output signal xr(t), and 6) the magnitude of its spectrum, Xr(f).

09.- Select a word or message, composed of a  symbol sequence, and determine at the receiver the two decimal digits (dits) transmitted in this digital communication.

10.- Proceed to calculate the information baud rate (Dr in symbols/second), symbol-bit rate (Sb in bits/symbol), the “info” gross bit rate (Rb in bits/second), and the code rate (Cr dimensionless), for  DTMF digital communication operation.

TAKE-HOME ASSIGNMENT GUIDELINES:

A.- This take-home work does not require a written result. It requires for each student group to send a .zip folder with the following items provided in the list below. There should be NOTHING else inside the requested .zip folder:

i.- Original, INEL4301_SYYY_GPZZ_MCA01_250825.docx

ii.- Original, INEL4301_SYYY_GPZZ_MCA01_250825pdf

iii.- Original, unaltered, dtmfad.wav and star_story.wav

iv.- Original, unaltered, inel4301s000gp00mca01.m

v.- One modified inel4301s000gp00sn00mca01.m per student.

Refers to the class section number: sxxx

Refers to the student’s group number: gpyy

B.- Name of .zip folder and the name of the e-mail subject:

INEL4301_SXXX_GPYY_MCA01

C.- Send e-mail, with the correct e-mail subject, and .zip file, with appropriate name, to the following e-mail address:

domingo.rodriguez1@upr.edu

STANDARD DEMERIT TABLE

| 01.- MATLAB m-script file does NOT execute | -05 pts. |
| --- | --- |
| 02.- Correct script file NOT included | -05 pts. |
| 03.- Missing INEL4301_SYYY_GPZZ_MAC01_260825.docx | -03 pts. |
| 04.- Missing INEL4301_SYYY_GPZZ_MCA01_260825.pdf | -03 pts. |
| 05.- Missing class section number sxxx | -02 pts. |
| 06.- Unzipped folder does NOT have same name | -02 pts. |
| 07.- Missing student’s group number gpyy | -05 pts. |
| 08.- Missing or incorrect e-mail subject | -05 pts. |

Late works will not be accepted!!!

%inel4301s000gp00sn00mca01

%Prof. Domingo Antonio Rodriguez

%Name of Student, sn01, DTMF_WORD, DITS, %XX of Contribution

%Name of Student, sn02, DTMF_WORD, DITS, %XX of Contribution

%Name of Student, sn03, DTMF_WORD, DITS, %XX of Contribution

%Name of Student, sn04, DTMF_WORD, DITS, %XX of Contribution

%Name of Student, sn05, DTMF_WORD, DITS, %XX of Contribution

%**************************Prof. D. Rodriguez***********************

%*********DSB-SC COMMUNICATIONS SYSTEMS AND GAUSSIAN NOISE**********

%*********ANALOG AND DIGITAL COMMUNICATIONS SIMULATIONS*************

%*******************************************************************

clear all                            %Clear working space

close all

%*******************************************************************

disp('******************************')

disp('Begin Simulation for Underwater Communications.')

disp('******************************')

%*******************************************************************

%********************Wanted Signal Input/Output*********************

%*******************************************************************

[sig,Fs]=audioread('star_story.wav');  %Get overall sampling frequency

save dsig.txt sig -ascii               %Save signal as dsig.txt file

load -ascii dsig.txt;                %Load ascii file dsig.txt

wsiz=length(dsig);                   %Get the length of the "dsig" signal

%*******************************************************************

%**************************Parameter Setting************************

%*******************************************************************

Ts=1/Fs;                          %Sampling time or temporal resolution

Nq=wsiz;                          %Number of samples of wanted signal

Vq=Nq*Ts;                         %Observation window or signal duration

fincq=1/Vq;                       %Frequency resolution of wanted signal

fkq=-(Fs/2):fincq:+(Fs/2)-fincq;  %Frequency axis of wanted signal

tq=0:Ts:Vq-Ts;                    %Time axis of wanted signal

%

fV=4000;                          %Cut-off frequency of wanted signal

Wn=(2*fV/Fs);                     %Normalized cut-off frequency

Lp=(1/567);                       %Proportionality factor

Nh=2*floor(Lp*Nq)+1;              %Low-pass filter length

hL=fir1(Nh,Wn);                   %Impulse response of low-pass filter

dsigL=conv(dsig,hL);              %Filtered wanted signal up to fV=4000 Hz

%

f1=4400;                             %Input interference frequency

f2=4900;                             %Input interference frequency

f3=5400;                             %Input interference frequency

g1=cos(2*pi*f1*tq);                  %Unwanted input signal

g2=cos(2*pi*f2*tq);                  %Unwanted input signal

g3=cos(2*pi*f3*tq);                  %Unwanted input signal

g=(1/80)*g1+(1/160)*g2+(1/240)*g3;   %Sum of unwanted signals

gmax=max(g);                         %Maximum value of sum

g=transpose((1/gmax)*g);             %Normalized interference signal

%********************************************************************

%*********************SIGNALS AND SYSTEMS MODELING*******************

fc=9900;                        %Carrier frequency

c=transpose((cos(2*pi*fc*tq))); %Carrier signal

wsig=dsigL(1:Nq,1);             %Wanted input signal

xm=wsig+g;                      %Sum of wanted and unwanted signals

xc=xm.*c;                       %Modulated wanted signal plus interference

%*********************************************************************

%*************************BAND-PASS FILTER****************************

Nb=1001;                           %Order of band-pass filter

Wb=[(2*4640)/Fs  (2*12640)/Fs];    %Vector of normilized cut-off freqs.

hB=fir1(Nb-1,Wb,'bandpass');       %Impulse response of band-pass filter

%*********************************************************************

%*************************CHANNEL INPUT SIGNAL************************

yci=conv(xc,hB);                    %Removal of unwanted interferences

Nyci=length(yci);                   %Order of channel input signal

tyci=0:Ts:(Nyci-1)*Ts;              %Time axis for yci(t)

fryci=1/(Nyci*Ts);                  %Frequency resolution for Yci(f)

fayci=-(Fs/2):fryci:+(Fs/2)-fryci;  %Frequency axis of wanted signal

%*********************************************************************

%*************************CHANNEL NOISE*******************************

nsig=randn(size(yci));          %Generation of AWGN signal: noise signal

nnsig=(1/max(abs(nsig)))*nsig;  %Normalized noise signal

nATT=sqrt(1/24000);                %Noise attenuation about -44 dB SNR

nsig=nATT*nnsig;                %Attenuated noise present in the channel

%*********************************************************************

%*************************CHANNEL OUTPUT******************************

yco=yci+nsig;           %Channel output signal with Gaussian noise

tqn=0:Ts:(length(yco)-1)*Ts;        %New time axis for output signal

cn=transpose((cos(2*pi*fc*tqn)));   %New carrier signal

yd=yco.*(2*cn);                     %Demodulation stage

%*********************************************************************

%*************************LOW-PASS FILTER*****************************

Nlow=663;              %Designed low-pass filter order

Wlow=(2*fV)/Fs;         %Normalized cut-off frequency

hlow=fir1(Nlow-1,Wlow); %FIR low-pass impulse response function

taxis_hlow=0:Ts:(length(hlow)-1)*Ts;

fres_hlow=1/((length(hlow))*Ts);

faxis_hlow=(-Fs/2):fres_hlow:(+Fs/2)-fres_hlow;

%*********************************************************************

%*************************RECEIVER OUTPUT*****************************

xr=conv(yd,hlow);       %Low pass filtering at the receiver

taxis_xr=0:Ts:(length(xr)-1)*Ts;

fres_xr=1/((length(xr))*Ts);

faxis_xr=(-Fs/2):fres_xr:(+Fs/2)-fres_xr;

%********************************SPECTRA*****************************

%********************************************************************

fwsig=fft(wsig);                %Spectrum of wanted signal (Fund. Reg.)

sfwsig=fftshift(fwsig);         %Spectrum of wanted signal (Princ. Reg.)

asfwsig=abs(sfwsig);            %Magnitude of spectrum of wanted signal

%

fxc=fft(xc);                  %Spectrum of modd. signal (Fund. Reg.)

sfxc=fftshift(fxc);           %Spectrum of modd. signal (Princ. Reg.)

asfxc=abs(sfxc);              %Magnitude of spectrum of modd. signal

%

fyci=fft(yci);                  %Spectrum of input signal (Fund. Reg.)

sfyci=fftshift(fyci);           %Spectrum of input signal (Princ. Reg.)

asfyci=abs(sfyci);              %Magnitude of spectrum of input signal

%

fyco=fft(yco);                  %Spectrum of output signal (Fund. Reg.)

sfyco=fftshift(fyco);           %Spectrum of output signal (Princ. Reg.)

asfyco=abs(sfyco);              %Magnitude of spectrum of output signal

%

fyd=fft(yd);                    %Spectrum of output signal (Fund. Reg.)

sfyd=fftshift(fyd);             %Spectrum of output signal (Princ. Reg.)

asfyd=abs(sfyd);                %Magnitude of spectrum of output signal

%

fnsig=fft(nsig);                %Spectrum of noise signal (Fund. Reg.)

sfnsig=fftshift(fnsig);         %Spectrum of noise signal (Princ. Reg.)

asfnsig=abs(sfnsig);            %Magnitude of spectrum of noise signal

%

fhlow=fft(hlow);                %Spectrum of IRF (Fund. Region)

sfhlow=fftshift(fhlow);         %Spectrum of IRF (Princ. Region)

asfhlow=abs(sfhlow);            %Magnitude of spectrum IRF

%

fxr=fft(xr);                    %Spectrum of xr(t) (Fund. Region)

sfxr=fftshift(fxr);             %Spectrum of xr(t) (Princ. Region)

asfxr=abs(sfxr);                %Magnitude of spectrum IRF

%******************************PLOTS*********************************

plot(tq,wsig);

axis([0 (length(wsig)*Ts) min(wsig)  max(wsig)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Wanted signal s(t) in the Time Domain')

grid

%

figure

plot(tq,g);

axis([0 (length(g)*Ts) min(g)  max(g)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Interference signal g(t) in the Time Domain')

grid

%

figure

plot(tq,xm);

axis([0 (length(xm)*Ts) min(xm)  max(xm)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Modulating signal xm(t) in the Time Domain')

grid

%

figure

plot(tq,xc)

axis([0 (length(xc)*Ts) min(xc)  max(xc)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Modulated Signal xc(t)')

grid

%

figure

plot(tyci,nnsig)

axis([0 (length(nnsig)*Ts) min(nnsig)  max(nnsig)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Normalized Noise Signal nn(t)')

grid

%

figure

plot(tyci,yci)

axis([0 (length(yci)*Ts) min(yci) max(yci)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Channel Input Signal yci(t)')

grid

%

figure

plot(tyci,yco)

axis([0 (length(yco)*Ts) min(yco)  max(yco)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Channel Output Signal yco(t)')

grid

%

figure

plot(fkq,asfwsig)

xlabel('Frequency in Hertzs')

ylabel('Magnitude')

title('Magnitude of Spectrum S(f) of Wanted Signal s(t)')

grid

%

figure

plot(fkq,asfxc)

xlabel('Freq in Hertzs')

ylabel('Magnitude')

title('Magnitude of Spectrum Xc(f)of Modulated Signal xc(t)')

grid

%

figure

plot(fayci,asfyci)

xlabel('Freq in Hertzs')

ylabel('Magnitude')

title('Magnitude of Spectrum Yci(f) of Channel Input Signal yci(t)')

grid

%

figure

plot(fayci,asfyco)

xlabel('Frequency in Hertzs')

ylabel('Magnitude')

title('Magnitude of Spectrum Yco(f) of Channel Output Signal yco(t)')

grid

%

figure

plot(fayci,asfyd)

xlabel('Frequency in Hertzs')

ylabel('Magnitude')

title('Magnitude of Spectrum Yd(f) of Demodulator Output yd(t)')

grid

%

figure

plot(taxis_hlow,hlow);

axis([0 (length(hlow)*Ts) min(hlow)  max(hlow)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Impulse Response Function of Receiver''s Low-Pass Filter')

grid

%

figure

plot(faxis_hlow,asfhlow)

xlabel('Frequency in Hertzs')

ylabel('Magnitude')

title('Magnitude of Spectrum of Impulse Response Function hlow(t)')

grid

%

figure

plot(taxis_xr,xr);

axis([0 (length(xr)*Ts) min(xr)  max(xr)]);

xlabel('Time in Seconds')

ylabel('Amplitude')

title('Receiver''s Low-Pass Filter Output Signal xr(t)')

grid

%

figure

plot(faxis_xr,asfxr)

xlabel('Frequency in Hertzs')

ylabel('Magnitude')

title('Magnitude of Spectrum of Receiver''s Output xr(t)')

grid

%

%*******************************OUTPUTS******************************

sound(wsig,Fs)  %Sound of Wanted Signal

disp('Sound of Wanted Signal')

pause(20)

sound(g,Fs)     %Sound of Interference Signal

disp('Sound of Interference Signal')

pause(20)

sound(xm,Fs)    %Sound of Modulating Signal

disp('Sound of Modulating Signal')

pause(20)

sound(xc,Fs)    %Sound of Modulated Signal

disp('Sound of Modulated Signal')

pause(20)

sound(yci,Fs)   %Sound of Channel's Input Signal

disp('Sound of Channel''s Input Signal')

pause(20)

sound(yco,Fs)   %Sound of Channel's Output Signal

disp('Sound of Channel''s Output Signal')

pause(20)

sound(xr,Fs)   %Sound of Channel's Output Signal

disp('Sound of Receiver''s Output Signal')

%********************************************************************

disp('******************************')

disp('Simulation ended succesfully.')

disp('******************************')